Middle East Technical University

Department of Electrical and Electronics Engineering

EE464: Static Power Conversion II

Term Project Final Report

Group Name: FlyBusri

Student 1: Elif Topaloğlu - 2443984

Student 2: Caner Aydın - 2374528

Student 3: Doruk Yazıcı - 2517225

Submission Date: 14.06.2024

Table of Contents

Introduction	3

Project Definition	3

Topology Selection	3

Validation of Design	5

General Design	5

Transformer Design	6

RCD Snubber Design	10

Open-Loop Simulation Results	11

## Introduction

The aim of the project is to design and implement an “Isolated DC-DC Battery Charger”, whose specs will be given in the upcoming parts of the report. In this report, design decisions will be investigated,, the transformer design for the selected topology will be explained, the circuit manufactured according to the simulations done will be clarified, closed-loop control loop of the design will be made clear, and the real-life test reults will be presented.

All of the documents, simulations and sources related to the design of the converter is pubhlished in the “GitHub Repository” of the project.

The project is compeled, but it is still open for improvements.

## Project Definition

An isolated DC-DC converter will be designed in whose topology can be selected in a way which suits the groups. The specs of the project are:

- Minimum Input Voltage: 20V

- Maximum Input Voltage: 40V

- Output Voltage: 12V

- Output Power: 60W

- Output Voltage Peak-to-Peak Ripple: 3%

- Line Regulation: 3%

- Load Regulation: 3%

- Closed-Loop Control  is a must.

## Topology Selection

To control the output current and voltage while achieving high efficiency in this project, a proper isolated DC-DC converter topology should be selected. In this section, several isolated converter topologies are considered and compared to select the most suitable one.

- Flyback Converter: This topology is one of the simplest designs that can be used in order to design an isolated DC-DC converter. It is advantageous due to uncomplicated design procedure since it only utilizes a single active switch. Moreover, a lower number of components are needed compared to the other types of topologies for this converter. A lower number of components makes sure that the design is cost effective compared to the other types of topologies which is an important advantage of this topology. Also, flyback converter is generally a good choice for low power applications which is smaller than 200W as it is the case for this project.

Apart from several handy advantages, there are multiple disadvantages of this topology as it single-ended type converter which means that it operates at a single quadrant of the BH curve. Also, it charges and discharges the inductors at different switching cycles, a gapped core is essential to increase the energy storage capacity. Thus, the main disadvantage can be poor transformer utilization. Moreover, due to high ripple currents at the input and output sides stemming from low inductance of the gapped core, larger capacitors should be utilized which is also a disadvantage for this topology.

- Forward Converter: This converter is mainly used for medium power applications in practice. One of the best advantages compared to the flyback is that the energy does not need to be stored as it is transferred in at the same cycle that is created. Thus, the magnetic core can be gapless, and the transformer utilization is better [2]. Using a gapless converter reduces the current ripple; thus, efficiency can be increased, and smaller rating components can be used which will probably lead to more compact design compared to the flyback converter.

The drawbacks compared to the flyback converter can be the increased cost as it uses an extra diode and an inductor, which is an important issue for low-budget projects. Moreover, as the energy is transferred for the same cycle, magnetizing current should be reset before the nest switching cycle which limits the maximum duty to 50%. If the transformer is not properly designed and controlled, higher duty cycle than 50% will lead to saturation of the core which makes a sensitive control system a must for this topology. Also, in the practical design, a third winding is added to protect the circuit from the leakage inductance effects. However, due to this winding, voltage across the primary switch is increased which results in higher voltage stress across the switch which is an important disadvantage which needs to be handled.

- Push-Pull Converter: The main application area consists of higher power applications since the power is distributed and handled by two active switches. Also, this topology differs from the previous two as it is double-sided which implies that the transformer is operating at two quadrants of the magnetic core which results in better utilization of the transformer. Therefore, this is a good choice for high power applications due to high efficiency.

However, due to the increased number of active switches, the total cost of this topology is fairly high compared to the other two. Also, control of these two active switches is more complex since the dead time should be arranged properly in order not to short circuit the source at the input. Moreover, a center-tapped transformer is used in this topology; thus, overall, the design procedure, when two active switches and transformer design is considered, is more complex. Finally, when two switches are off, voltage stress across the two switches is still quite high, which may cause problems when the switch is not properly selected and cause heating problems as well as increased losses due to possible high on resistances.

- Half-Bridge and Full-Bridge Converters: These two topologies are also used for high power applications, generally higher than push-pull converters. The advantage of these two compared to the push-pull converter is that the voltage stress across the switches is decreased. Moreover, these are also double sided; however, they have single primary winding which makes sure that the transformer is utilized better compared to the push-pull converter [2].

The disadvantage of these topologies is that half-bridge cost is slightly more than the push-pull converter. For the full-bridge converter, the cost is considerably higher as it includes 4 active switches which also complexes the design and the controllers. These topologies become an overdesign for low power applications.

Considering all of the mentioned topologies, and the low output power requirement of the project, push-pull, half-bridge and low-bridge converters are found to be overcomplicated for this specific application. Also, as the cost and complexity are important points for this project, flyback converter topology is found to be applicable and sufficient. In the nest section, a flyback converter will be designed and presented.

## Validation of Design

### General Design

Figure 1. Flyback Converter Topology

For this project, as explained in the previous section, a flyback converter has been selected. The general topology schematic is given below. In order to avoid sudden current changes due to the leakage inductance, and the additional losses occuring due to the ringint of voltage on the MOSFET switch and secondary side diode, an RCD snubber design is added to ensure the safety of the design and the switch.

As a closed-loop control is mandatory for the project, a controller needs to be used to ensure that the voltage at the output is 12V continuously. For this reason, an analog controller will be utilized. Due to the low side placement of the MOSFET, driver circuitry will be simpler and there will be no need for a bootstrap circuit, only a MOSFET gate driver integrated circuit will be utilized, as the output current of the analog controller may not be able to operate open the gate of the MOSFET. To ensure the isolation boundary is preserved, an optocoupler is used for the closed loop control. In the following section, analytical calculations for the transformer design and RCD snubber design will be presented.

### Transformer Design

To start with the magnetic design of the topology, duty cycle should be decided to calculate the corresponding turns ratio. In flyback converters, duty cycle can obtain values between zero and one. However large duty cycle values do not assure a good performance. If the input voltage is low, which implies a high duty cycle, conduction losses will increase due to increased conduction time. Moreover, for a poorly designed transformer, large duty values may result in core saturation due to decreased allocation time for the flux reduction inside the core. Both situations decrease the efficiency of the converter which is unwanted. Thus, maximum duty cycle is selected as 0.5 for this design. When the maximum duty cycle is selected, maximum turns ratio can be calculated using the following formula below. The important point in this calculation is that output voltage is taken as 13V to consider the voltage drop across the output diode and to stay within the predefined duty cycle limits. For ease of manufacturing and simplicity, turns ratio is selected as 1. For the selected turns ratio, duty cycle for both input voltages are calculated below.

N1N2≤Vin*D(Vout+1)*1-D
N1N2≤20*0.513*(1-0.5)≅1.54 turns

D=1VinVout+1+1
Dmax=1(2013+1)=0.393≅0.4  &  Dmin=14013+1=0.245

Table 1. Turns Ratio Selection and Duty Cycle Limits

| Turns Ratio (N1/N2) | 1 |
| --- | --- |
| Duty Cycle Range | 0 to 0.5 |
| Maximum Duty Cycle | 0.393 |
| Minimum Duty Cycle | 0.245 |

At this point, a target efficiency should be selected in order to move on to further parameter calculation. Target efficiency is selected as:

ηtarget=0.8

After determining the efficiency target, input power can be calculated:

Pin=Poutηtarget=60W0.8=75W

Knowing the limits of the duty cycles, the average current flowing on the magnetizing inductance during on period when the input voltage is in its low limit can be calculated as [3]:

IEDC=PinVin,min⋅Dmax=75W20V⋅0.4=9.375A

At this point, another design decision should be given which is the value for KRF, which denotes “current ripple factor”. Here, the current ripple factor is selected as KRF=0.33 which is within the reasonable limit as stated in the design document. Thus, the current ripple is calculated as below.

ΔI=2⋅KRF⋅IEDC=2⋅0.33⋅9.375=6.1875A

Then:

IEDC,max=9.375+6.18752=12.47A≅12.5A
 IEDC,min=9.375-6.18752=6.28A≅6.3A

This shows that the peak current on the inductor and hence the MOSFET is approximately 12.5A.

As the minimum primary inductor voltage, the switching frequency, the duty cycle at the minimum voltage and current ripple are known, the inductance of the primary inductor can be easily calculated using the inductor equation:

VL=L⋅didt

Considering the on period, the same equation can be converted to the equation below. Assuming a higher current ripple factor for the higher value of the input voltage, the same equation can be written as:

VL=L⋅ΔiD⋅Ts→Lmin=VL⋅D⋅TsΔi=Vin,max⋅Dminfsw⋅Δi=40V⋅0.245100kHz⋅6.1875A=19.79μH

Leaving a safety margin which will in fact lower the voltage ripples on the magnetizing inductance, the primary inductance of the transformer is selected as:

Lm=20μH

At this point, the core material is decided. In order to increase the energy storage capacity for the flyback converter, the core should have an airgap. After some research, the core found is “B66363G0500X187” which is an N87 type ETD39 ferrite core from TDK Electronics. It has a built-in 0.5mm airgap so that when two of them are used, a total of 1mm airgap will be obtained. Datasheet of the core provides some valuable information considering the airgaps:

AL=196nHT2 & μr=115 & Ie=92.2mm

To find the number of turns in the primary side:

20μH=196nHT2⋅N2→N=10.1 Turns

The turn number is required as an integer and considering the safety left at the inductance value, the turn number selection is made as:

N=11 Turns→Lm=23.71μH

As the turns ratio is selected as 1, the secondary side will have the same number of turns.

Then, to find the magnetic flux density (B) inside the core, the magnetic field intensity is found using Ampere’s Circuital Law:

N⋅Imax=Hmax⋅dL→11⋅12.5A=Hmax⋅92⋅10-3m

Hmax=1490.50Am

Then:

Bmax=μ0⋅μr⋅Hmax=4π⋅10-7⋅115⋅1355.75Am=0.215T

Considering the information given in the datasheet of N87 materials by TDK Electronics, the found value is far away from the saturation of the core, which is given approximately as 0.4T.

Considering the operating frequency of 80kHz, the cable is selected as “AWG26” since it offers an operating frequency of 107kHz for maximum skin depth. To be able to calculate the number of parallelled conductors, the RMS value of the current flowing through the transformer should be found. RMS value for a triangular wave with an offset can be calculated using the formula given below [4].

IRMS=IEDC*1+112*ΔIIEDC 2
IRMS=9.375*1+112*6.18759.375 2 ≅9.54A

The AWG26 cable can carry 0.361Aconductor so that the number of conductors that should be wound together is as follows.

9.540.361≅26.44→27 will be used

One AWG26 cable has a conductor cross section of 0.128 mm2 but considering the isolation on them, this value will be taken as 0.14 mm2 for more realistic calculations. Assuming same number of conductors will be used for both the primary and secondary sides, the total area that will be occupied by the conductors can be calculated as:

2⋅0.14mm2⋅27⋅10=75.6 mm2

TDK Electronics recommends coil formers B66364B1016T001 or B66364W1016T001 to be used with the selected core which offer 178mm2 of window area.

This results in a fill factor of:

Fill Factor=75.6 mm2178 mm2≅0.42

which is a reasonable value.

In the table of AWG cables, the resistance of the AWG26 cable is given as 133.8568Ω1000m. The mean path length of the coil formers is given as 69mm. For the primary or the secondary side, the total DC resistance can be calculated as:

RDC=10⋅69⋅10-3m⋅133.8568Ω1000m⋅120=4.618⋅10-3Ω

The AC resistance can be calculated as follows:

RAC=ρ⋅lAeff & Aeff=δ⋅π⋅d & δ=ρπ⋅f⋅μ

δ=1.678⋅10-8π⋅80⋅103⋅4π⋅10-7⋅0.999991=230.5⋅10-6m

Since the skin depth is larger than the actual cross section of the cable, effective area is taken as the cable’s cross section area assuming that the current will flow uniformly.

Aeff=230.5⋅10-6m⋅π⋅0.40386⋅10-3m=292.45⋅10-9m2

RAC=1.678⋅10-8⋅10⋅69⋅10-30.128*10-6⋅120=4.52*10-3Ω

It can be seen that the AC and DC resistances of AWG26 cable at the specified frequency is almost same. It is expected that AC resistance should be higher than the DC resistance, which is not the case, and this will probably stem from the given resistance multiplier in the datasheet for the DC resistance.

The total copper loss can be calculated as:

Pcu=2⋅IDS,RMS2⋅RDC=2⋅9.542⋅4.687⋅10-3=0.85W

As stated before, the core material is N87. Reading the datasheet of the “N87 SIFERRIT Material”, the core loss values for some frequency and Tesla values can be found. In the datasheet, the core loss for 80kHz and 200mT at 100°C is given approximately as 280kWm3. The datasheet of the B66363G0500X187 core states that the volume of a core is 11500mm3. Then:

Pcore=280kWm3⋅11500⋅10-9mm3=3.22W

The total loss of the transformer can be estimated as:

Ploss=Pcore+Pcu=3.22W+0.85W=4.07W

The design does not require new iterations as the values are in acceptable limits, and most importantly the cores are found in a distributor in Turkey, who can bring them with all the additional materials in 10 days maximum. For the wire, instead of wounding tons of AWG cables together, the “Litz Wire” is used which is supplied by “METU Powerlab”, greetings to them.

### RCD Snubber Design

For the RCD snubber design, a handbook is found from Fairchild. A proper RCD snubber should be utilized in order to avoid the devastating effect of leakage inductance. Snubber values are calculated below using the formulas presented in the handbook.

Rsb=Vsb2*(Vsb-nVo)0.5*Lleakage*Ipeak2*Vsb*fs

Csb=VsbRsn*ΔVsb*fs

These values should be obtained using simulation software. Simulation results will be presented in the following part; however, using the values obtained from the simulation, snubber values can be calculated using the given formulas above. The calculations are presented below.

Rsb=302*(30-12)0.5*0.25*10-6*11.52*30*80*103=408 Ω

Csb=30408*2.9*80*103=317 nF

In the following section, using the analytical calculations, simulations results will be presented in order to validate the design.

### Open-Loop Simulation Results

Figure 2. Circuit Schematic of the Open-Loop Flyback Converter

To be able to validate the various design decisions taken, an open-loop simulation is created as the first step. The simulation is run for input voltages of 20V and 40V separately, and the results will be presented separately also. The circuit schematic can be seen in Figure 2.

First of all, the input voltage is set as 20V and the duty cycle is set as 0.4. It should be noted that all of the measurements are taken in the steady state, and with the tools of LTSpice.

Figure 3. Output Voltage and Current Measurements for Vin = 20V

The average output voltage is measured as 12.141V, and the average output current is measured as 5.059A which results in an average output power of 61.398W. The average input power is measured as 75.738W which results in an efficiency of 0.811.

Figure 4. Vds Measurements on the MOSFET for Vin = 20V

The ripple on the output voltage is equal to 249.85mV at steady state which results in an output voltage regulation of 2.06%.

The maximum voltage measured on the MOSFET is 55.2V at the steady state. During startup, the voltage on the MOSFET increases to 90V for an instance.

Figure 5. Ids Measurements on the MOSFET for Vin = 20V

Figure 6. Forward Current on the Output Diode for Vin = 20V

The current on the MOSFET increases to 11.51A from 7.63A during the ON period. During startup, the current on the MOSFET increases to 27A for an instance. These maximum values of current and voltage on the MOSFET are considered when the components are chosen.

The current on the output diode decreases to 6.64A from 10.95A during a cycle. During startup, the current on the output diode reaches 27A for an instance.

Figure 7. Voltage on the Output Diode for Vin = 20V

The output diode should be able to stand a reverse voltage of 68.54V. During startup, this value reaches up to 79V. These maximum values of current and voltage on the output diode are considered when the components are chosen.

Figure 8. Output Voltage and Current Measurements for Vin = 40V

Then, the input voltage is set to 40V and the duty cycle is set as 0.245.

The average output voltage is measured as 12.101V, and the average output current is measured as 5.042A which results in an average output power of 60.994W. The average input power is measured as 81.95W which results in an efficiency of 0.744.

Figure 9. Vds Measurements on the MOSFET for Vin = 40V

The ripple on the output voltage is equal to 161.26mV at steady state which results in an output voltage regulation of 1.33%.

Figure 10. Ids Measurements on the MOSFET for Vin = 40V

The maximum voltage measured on the MOSFET is 71.3V at the steady state. During startup, the voltage on the MOSFET increases to 107V for an instance.

The current on the MOSFET increases to 10.86A from 6.16A during the ON period. During startup, the current on the MOSFET increases to 27A for an instance. These maximum values of current and voltage on the MOSFET are considered when the components are chosen.

Figure 11. Forward Current on the Output Diode for Vin = 40V

Figure 12. Voltage on the Output Diode for Vin = 40V

The current on the output diode decreases to 4.36A from 9.32A during a cycle. During startup, the current on the output diode reaches 27A for an instance.

The output diode should be able to stand a reverse voltage of 100.59V. During startup, this value reaches up to 101V. These maximum values of current and voltage on the output diode are considered when the components are chosen.

The open-loop simulation is a good first step towards the design process, but a closed-loop simulation should be implemented for a better visualization of the real-world design.

### Closed-Loop Simulation Results

For a real-world application, a closed-loop system should be designed so that the system can respond to changes in the input and regulate a better output even if the output load changes also.

It was stated that the bonus for using an analog IC is aimed so that a closed-loop design using an analog IC is generated. For the design, the “WEBENCH Power Designer” tool from the Texas Instruments. From many options generated, the one that uses UC3845 Analog IC is selected as the mentioned IC can be found easily in Turkey, and it fits the design considerations. A critical point is that the series of ICs “UC384x” are very similar in terms of properties. The onbly difference fo UC3845 is that the maximum duty cycle it can generate at its output pin is 50%. As  mentioned before, the maximum duty cycle for the application is less than 50%, so that the other ICs of the same family can be used, according to the stocks that are present in Turkey..  Another advantage of the circuit is that by changing some of the feedback resistances, the output voltage can be regulated easily, if there is a need for that. Also, the feedback loop can be updated.

The reference design obtained from the tool can be seen in Figure 13.

Figure 13. Reference Design of the Flyback Converter with a UC3845 IC

Then, the design is generated in LTSpice for simulations. During the generation of the circuit, the startup resistors and the auxiliary winding are not included, as we will be powering up the UC3845 with an LM2596 directly from the input. It is tested that the LM2506 can work in the input voltage range of 20-40V, and it can supply the required voltage and current to UC3845.

Figure 14. Closed-Loop Circuit Schematic using an UC3845 IC

The schematic of the circuit can be seen in Figure 14.

The simulation consists of the isolated output voltage feedback loop, the RCD clamp, the RC snubber for the switch, output LC filter, and leakage inductances to be able to generate a circuit as close as possible to a real-life application.

The isolated feedback is obtained using an optocoupler. The feedback is generated with a 10V zener diode. The feedback signal generated is sent back to the UC3845 IC.

The RCD clamp is used to decrease the voltage stress on the switching MOSFET. Also, an RC snubber is placed in parallel with the secondary side diode.

The output LC filter helps the circuit to obtain an output which has less ripple on it.

The circuit will be operated both in inputs of 20V and 40V, and the results will be presented.

Figure 15. Output Current and Voltage Measurements of the Closed-Loop Circuit for Vin = 20V

First, the input voltage is set to 20V.

Figure 16. Vds Measurements on the MOSFET for Vin = 20V

Figure 15 represents the output current and voltage of the flyback converter. The response reaches a steady state approximately at 6 milliseconds. It can be seen that the responses include very low ripple at the steady state. The average output current at the steady state is measured as 5.065A and the average output voltage at the steady state is measured as 12.157V. The maximum and minimum values that the output voltage oscillate between are 12.164V and 12.150V respectively. This results in an output voltage regulation of 0.11%. It can be said that the result is in a safe margin as in a real-life application, many non-idealities will be disturbing the circuit, and the results may differ from an ideal scenario. The average input power is 80.695W while the average output power is 61.579W which results in an efficiency of 0.763.

Figure 17. Ids Measurements on the MOSFET for Vin = 20V

The maximum voltage measured on the MOSFET is 53.2V at the steady state. During startup, the voltage on the MOSFET increases to 117V for an instance.

Figure 18. Voltage on the Output Diode for Vin = 20V

The current on the MOSFET increases to 12.28A from 9.012A during the ON period. During startup, the current on the MOSFET increases to 52.8A for an instance. These maximum values of current and voltage on the MOSFET are considered when the components are chosen.

Figure 19. Forward Current on the Output Diode for Vin = 20V

The output diode should be able to stand a reverse voltage of 90V. The ringing on it does not induce a power loss as no current passes through it.

The current on the output diode decreases to 6.81A from 10.39A during a cycle. These maximum values of current and voltage on the output diode are considered when the components are chosen.

Figure 20. Output Current and Voltage Measurements of the Closed-Loop Circuit for Vin = 40V

Then, the input voltage is set to 40V.

Figure 21. Vds Measurements on the MOSFET for Vin = 40V

Figure 20 represents the output current and voltage of the flyback converter. The response reaches a steady state approximately at 6 milliseconds. It can be seen that the responses include very low ripple at the steady state. The average output current at the steady state is measured as 5.185A and the average output voltage at the steady state is measured as 12.444V. The maximum and minimum values that the output voltage oscillate between are 12.45V and 12.438V respectively. This results in an output voltage regulation of 0.10%. It can be said that the result is in a safe margin as in a real-life application, many non-idealities will be disturbing the circuit, and the results may differ from an ideal scenario. The average input power is 90.603W while the average output power is 64.511W which results in an efficiency of 0.712.

Figure 22. Ids Measurements on the MOSFET for Vin = 40V

The maximum voltage measured on the MOSFET is 69.5V at the steady state. During startup, the voltage on the MOSFET increases to 124V for an instance.

Figure 23. Voltage on the Output Diode for Vin = 40V

The current on the MOSFET increases to 11.95A from 7.637A during the ON period. During startup, the current on the MOSFET increases to 87.5A for an instance. These maximum values of current and voltage on the MOSFET are considered when the components are chosen. Also, the peak one-cycle ratings are considered.

Figure 24. Forward Current on the Output Diode for Vin = 40V

The output diode should be able to stand a reverse voltage of 100V. The ringing on it does not induce a power loss as no current passes through it.

The current on the output diode decreases to 4.76A from 9.41A during a cycle. These maximum values of current and voltage on the output diode are considered when the components are chosen.

Overall, it can be seen that the design can be implemented with a proper choice of the components of the circuit.

### Component Selection

The components are mainly picked from the local suppliers such as “ozdisan.com” and “direnc.net”. The most important components for the operation of the converter are the semiconductors such as the switching MOSFET, output diode and clamp diode. The other passive parts and ICs are also found from the mentioned suppliers.

The MOSFET selected is “IRF540NPBF” produced by International Rectifier. It is a 33A 100V N-Channel MOSFET in TO-220AB package with a 44mΩ drain-to-source resistance.

The diode selected for the output and the RCD clamp are same. BD10200CS_S2_00001 is a Schottky diode produced by Panjit in DPAK packaging which can pass through 10A and hold a reverse voltage of 200V. The forward voltage of the diode is equal to 0.9V, which is considered in the design as the additional  1V output that we have considered in the transformer design section.

A 10V zener diode will be used in the feedback generation circuit.

UC3843 will be used as the controller of the circuit. When the datasheet of the controller is inspected, it can be seen that as well as UC3843; UC3844, and UC3845 can be also used as the properties are exactly same except the operating temperature and duty cycle which does not imply a problem for the project.

The optocoupler used is a PC817 which can be easily found in Turkey.

### Transformer Tests

Figure 26. Manufactured Transformer Design

For starters, to verify that the magnetic design is accurate and achievable, transformer windings are manufactured and wound to the core itself. The first design of the transformer is presented below.

As it is calculated before, the inductance of the primary winding should be approximately 20µH. To achieve this, primary and secondary windings should be 10 turns. After the transformer is manufactured, some tests are conducted to evaluate the parameters of the transformer.

Figure 26.  LCR meter measurements: a) secondary side is open circuited, b) secondary side is short circuit.

Initially, the primary side of the transformer is connected to the LCR meter, and the inductance and resistance are measured while the secondary is open circuited. Corresponding measurements are given in the figure below.

Figure 27. Equivalent Circuit of a Transformer

For the test setup, the equivalent circuit of the transformer is assumed to be as below.

As the turns ratio of the transformer is 1 in this case, leakage for the primary (L1) and the referred secondary leakage inductance (L2) are equal with small possible deviations. For the first test, the LCR meter is connected to the primary side of the transformer while the secondary size is open circuited. Therefore, measured inductance is equal to sum of the leakage inductance and the magnetizing inductance.

L1+Lm=24.504 μH

For the second test, the secondary side is short circuit. Thus, the measured inductance includes both the primary leakage summed with parallelly connected secondary referred leakage with the magnetizing inductance.

L1+L2//Lm=L1+L1//Lm=0.5μH

Using the two equations, leakage inductance and the magnetizing inductance can be calculated. The results show that the magnetizing inductance is significantly higher compared to the leakage inductance. Thus, the second measurement where the secondary side is short circuit, one can assume that the parallel branch does not demand significant current which implies that the measured inductance is approximately equal to twice of the leakage inductance. Lastly, the ratio of the leakage to the magnetizing inductance is calculated as approximately 1% which is reasonable.

L1≅0.2513 μH
Lm≅  24.2527 μH
L1Lm%=0.251324.2527*100≅1.04%